Analysis of LTP-IE using Igor Pro software.

This procedure needs the installation of two Igor procedures: “Neuromatic v2_00” and “Spike tools”. “Neuromatic” can be obtained at http://www.neuromatic.thinkrandom.com/ and the code for “spike tool” is available at the end of this procedure.

- In Neuromatic menu, click import waves, new data folder, continue, and select file to open.

- Spike count:

  - In Neuromatic window, select Spike tab and set the t_bgn value as -inf and t_end as inf. Set the threshold value to detect every spike in a characteristic sweep.

  - Run spike count by clicking on all waves button

  - Display results while computing?  select yes

  - Display delay select 0

  - click continue

  - In Spike tools menu, select Spike count

  - Click continue

  - In the generated table, the second column gives the sweep number and the third column gives the spike count in the corresponding sweep.

Resting membrane potential and input resistance automated measurement.

3.1 in Neuromatic window, select Stat tab and set t_bgn and t_end values as the beginning and the end of the hyperpolarizing current used to test input resistance.

3.2 set the win0 measurement to min in order to measure the minimal membrane potential value during the current injection.

3.3 check the Bsln box to set cursors positions where to measure the baseline (in green)

3.4 set the win1 measurement to Avg in order to measure the resting membrane potential value.

3.5 set t_bgn and t_end values to select part of the sweep where you want to measure resting membrane potential (30ms just before the beginning of the test current injection).

3.6 Click all waves to start measurements. When finished, click on continue in the dialog box.

3.7 In the generated table, the first column gives the sweep number, the third gives the amplitude of the hyperpolarization provoked by the short step of negative current (that allows to calculate the input resistance), and the sixth gives the resting membrane potential.

Spike Tools code :

#pragma rtGlobals=1		// Use modern global access method.

Menu "spike tools"

"spike count", spkfreq()

End

Function spkfreq()

String set

variable tbegin, tend

prompt set,"set as declared by ",popup,Wavelist("SP_RX_*",";","")

DoPrompt "Set to Analyse for spk count",Set

string RX,RY,SWP,FRQ

RX=set

RY=set

FRQ=set

SWP=set

RY[3,4]="RY"

SWP[0,4]="SWP"

FRQ[0,4]="FRQ"

variable i,i_end,swpnb,freq,j

i_end=numpnts($RY)

make/O/N=1 $SWP

make/O/N=1 $FRQ

duplicate/O $RX dRX

duplicate/O $RY dRY

duplicate/O $SWP dSWP

duplicate/O $FRQ dFRQ

i=0

j=0

dSWP[j]=dRY[i]

if (numtype(dRX[i])==0)

dFRQ[j]=1

else

dFRQ[j]=0

endif

i=1

do

if (dRY[i]==dRY[i-1])

dFRQ[j]+=1

else

j+=1

InsertPoints j,1, dFRQ

InsertPoints j,1, dSWP

dSWP[j]=dRY[i]

if (numtype(dRX[i])==0)

dFRQ[j]=1

else

dFRQ[j]=0

endif

endif

i+=1

while (i<i_end)

j+=1

InsertPoints j,1, dFRQ

insertPoints j,1, dSWP

dFRQ[j]=freq

dSWP[j]=dRY[i-1]

duplicate/O  dSWP $SWP

duplicate/O  dFRQ $FRQ

killwaves dRX dRY dSWP dFRQ

edit/W=(5.25,271.25,348,487.25) $SWP, $FRQ

•Display $FRQ vs $SWP

•ModifyGraph mode=5;DelayUpdate

•Label left "Nb of spikes";DelayUpdate

•Label bottom "Trace #"

end