Each image is a screenshot from the section 3. Data processing and analysis of the protocol. The description above each image matches the step identifier and its description. Use this image guide together with the protocol.

3.1.1.	Download and uncompress the repository being used (own or sample) if it has not yet been downloaded.

3.1.2.	Ensure the scripts 1.unpacking_sheets.m, 2.data_merge_and_process.m, 3.3D rotational trajectory.m, and the folder models are inside the downloaded repository in the folder Octave, and move the data files downloaded from step 2.3.2 and 2.3.3 to this same folder where the octave scripts are. NOTE: Any existing files already in the folder with the same names as the newly written files might be overwritten. Rename or move the files to another folder accordingly.

3.1.3.	Open the script 1.unpacking_sheets.m with the GNU Octave Launcher. In the Editor tab, run the script by clicking on the green Save file and run button, to unpackage the data to a more readable structure.

3.1.4.	Two prompts will appear, one after the other. Input the name of the downloaded file inside the first prompt and the name of the unpackaged file inside the second field. Alternatively, leave both prompt fields blank to use the default names meant for the sample files included. Wait a few minutes (depending on the volume of data) for a popup informing the user that the process is complete and the new file has been written.

3.1.5.	Open and run the script 2.data_merge_and_process.m to merge the data from both, eye tracker and iRT.

3.1.6.	Four prompts will appear. Input the sessionID value, taskID value (both from the iRT data table), unpackaged iRT data filename (written in step 3.1.5), and eye tracker data filename (exported in step 2.3.2) or leave all of them blank to use the default values.

NOTE: After some minutes, a help popup will indicate that the script completed the computation and the names of the files used and created. Three sample plots of angular disparity will appear during the script process: a simple plot, a plot with colored gaze data, and a plot with pupil diameter data. The two files created are output merge X Y.xlsx and output jmol console X Y.xlsx, where X is the sessionID value, and Y is the taskID value, both written at the beginning of step 3.1.6.

3.2.1.	Open and run the script 3.3D rotation trajectory.m.

3.2.2.	Three prompts will appear. Input the sessionID value, taskID value, and unpackaged iRT data filename, or leave them blank to use the default values.

NOTE: A 3D graph will appear. The rendered graph is the 3D rotation trajectory of the specified session and task.

3.3.1.	To replay the participant’s task interaction, first, go to the interactive task webpage, start the test (showing both 3D models), move the mouse pointer at the top-right corner of the screen until the mouse icon changes to text, as depicted in the Supplemental File 2, and then click on the invisible Debug text, enabling the debug mode.

3.3.2.	From the buttons that appear between the models, click on the timerStop button to interrupt the task and click on the console button to open the JSmol console of the model on the right. If the task from the interaction of interest was not the first one, click on the numbered buttons inside the top debug area to change the task that is being shown on the screen.

3.3.3.	Open the file output jmol console.xlsx and copy the entire page of Jmol commands.

NOTE: Each page contains commands for a different scene or animation.

3.3.4.	Inside the JSmol console, paste the list of commands copied and click the Run button or press Enter on the keyboard to execute it.

3.3.5.	If desired, generate a .gif animation. Write the command capture "filename" SCRIPT "output" inside the JSmol console, where filename is the name of the .gif file to be created and output is the entire list of commands copied in step 3.3.3, keeping both inside the double quotation marks.