“Kymo”

//Macro for creating Kymograph

//Lai Ding  HNDC Enhanced Neuroimaging Core

//Image is single channel. user should examine is there is any spatial shift in thte time series.

//user should also do backsubtraction, smooth if necessary before running the macro

//you should open the raw image in ImageJ, then run the macro.

//all result files will be saved in the same folder

//assume kymoline is less than 5000 pixels.

//the raw data is rescaled t opixel unit, you will input the pixelscale in the TrackMacro for analysis

//result files

//filename_kymoline.txt   raw kymoline X Y coordinates  XY coordinates in pixel unit

//the distance between each pair of coordinates are 1 pixel.

//filename_kymo.tif    kymo image for next macro

//filename_kymolinbe.zip   user drew kymoline.

// initialize ImageJ

run("Colors...", "foreground=white background=black selection=yellow");

run("Options...", "iterations=1 count=1 black edm=Overwrite");

// raw data parameters

var dir, name, bit, t_dim, kymolength;

var linewidth=3;  // width of user drew line, take more average.

//global recording parameters.  assume the track line is less than 2,000 points

var cor_X=newArray(5000);

var cor_Y=newArray(5000);

// get rawdate's folder and filename

dir=getInfo("image.directory");

name=getInfo("image.filename");

bit=bitDepth();

t_dim=nSlices;

//create and save kymo image

kymo_make();

run("Close All");

//-----------------------------------------   functions ----------------------------------

function kymo_make()

{

rename("raw");

//run("Median...", "radius=1 stack");

run("Properties...", "channels=1 slices=1 frames="+t_dim+" unit=pixel pixel_width=1 pixel_height=1 voxel_depth=1 frame=[0 sec] origin=0,0");

run("Z Project...", "projection=[Max Intensity]");

userdrawline();

readXYcoordinates();

createKymo();

}

function createKymo()

{

if(bit==8)

newImage("kymo", "8-bit Black", kymolength, t_dim,1);

if(bit==16)

newImage("kymo", "16-bit Black", kymolength, t_dim,1);

selectImage("raw");

setBatchMode(true);

for(t=0;t<t_dim;t++)

{

for( j=0;j<kymolength;j++)

{

selectImage("raw");

setSlice(t+1);

tmp=getPixel(cor_X[j], cor_Y[j]);

selectImage("kymo");

setPixel(j,t,tmp);

}

}

selectImage("kymo");

saveAs("Tiff", dir+name+"_kymo.tif");

close();

setBatchMode(false);

updateDisplay();

}

function userdrawline()

{

setTool("polyline"); roiManager("reset");

run("Line Width...", "line="+linewidth);

title = "User draw selection";

msg = "User draw selection,add to the ROI Manager,  then click \"OK\".";

waitForUser(title, msg);

roiManager("Select", 0);

run("Interpolate", "interval=1");

saveAs("XY Coordinates", dir+name+"_kymoline.txt");

roiManager("save", dir+name+"_kymoline.zip");

roiManager("reset");

}

function readXYcoordinates()

{

data = File.openAsString(dir+name+"_kymoline.txt");

datapoints = split(data, "\n");

kymolength=datapoints.length;

for (i = 0; i < (datapoints.length); i++)

{

tmp = split(datapoints[i], "\t");

cor_X[i]=tmp[0];

cor_Y[i]=tmp[1];

}

}

“Track”

// Track Macro

// work on Kymograph image created by Kymo macro.

// user open filename__kymo.tif Image, the nrun the macro, all result fiels save in the same folder

// require current folder contains filename, filename_kymoline.txt,

// filename_select.tif is optional. if file exists, the macro will show already selected line and user can input starting puncta_count indexd

// assume kymoline is less than 5000 pixels long

// only record the point user click.

// still show the track on "select.tif"

// raw txt file only show (x,t) with user clicked point.

// initialize ImageJ

run("Colors...", "foreground=white background=black selection=yellow");

run("Options...", "iterations=1 count=1 black edm=Overwrite");

run("Line Width...", "line=1");

// parameters

var dir, name, tmpname, bit,  kymolength, t_dim, boxsize;

var cor_X=newArray(5000), cor_Y=newArray(5000);

var click_index;

// start : start time point of selected puncta track,  t_duration  number of time points for the puncta track

var start=0,  end=0, t_duration=0;

var total_puncta=0, puncta_count, flag, colorflag;

// X Y position of each track at time point, for selected point in kymo graph at (x,t),

//the corresponding xy position in real image is posX[t]=cor_X[x], posY[t]=cor_Y[x]

var T=newArray(5000), KP=newArray(5000);

// X Y position of each track at user clicked point, for selected point in kymo graph at (x,t),

//the corresponding xy position in real image is posX[i]=cor_X[KP[i]], posY[i]=cor_Y[KP[i]]

var posX=newArray(5000), posY=newArray(5000);

// x y position of individual puncta track in Kymo graph.

// x value is used to extract X Y position in real image based on cor_X. cor_Y array

var tmpX=newArray(5000);

// for use of mouse cursor track

leftButton=16;  rightButton=4;  shift=1;  ctrl=2; alt=8; x2=-1; y2=-1; z2=-1; flags2=-1; logOpened = false;

//------------------------------- start main function -------------------------------------

dir=getInfo("image.directory");

name=getInfo("image.filename");

t_dim=getHeight(); kymolength=getWidth();

run("Brightness/Contrast...");

run("Enhance Contrast", "saturated=0.35");

// read XY coordinates.  XY coordinates file is names as filename_kymoline.txt.

readXYcoordinates()

// initialization before each puncta selection.

start_up();

// input analyze parameters

parameterinput();

flag=true;

while( flag )

{

click_index=0;

kymo_cal();

// wait for user to decide if

title = "Choice";

msg = "Check the show window and mito window to see if track result, then click OK \"OK\".";

waitForUser(title, msg);

flag1=getBoolean("want save this mito track as Puncta Track #"+puncta_count+" ?");

if( flag1 )

{

savepuncta();

puncta_count++;

if( colorflag=="red")   colorflag="green";

else if( colorflag=="green")   colorflag="blue";

else colorflag="red";

open(dir+name+"_select.tif");

rename("select");

}

else

{

//close "select.tif, reopen it.

selectImage("select"); close();

if( File.exists(dir+name+"_select.tif") == 1 )

open(dir+name+"_select.tif");

else

{

// create "select" image"

selectWindow(name);

run("Duplicate...", "title=1");

selectWindow(name);

run("Duplicate...", "title=2");

selectWindow(name);

run("Duplicate...", "title=3");

run("Merge Channels...", "c1=1 c2=2 c3=3");

}

rename("select");

}

roiManager("reset");

flag=getBoolean("For another track yes/ quit no ?");

}

selectWindow("select");

saveAs("Tiff", dir+name+"_select.tif");

run("Close All");

//---------------------------------------  function line ------------------------------------------------------

// input parameters for calculation

function parameterinput()

{

Dialog.create("Parameter");

Dialog.addNumber("puncta_count start with:", 1);

Dialog.addChoice("Show Box Size", newArray(1, 3, 5));

Dialog.show();

puncta_count = Dialog.getNumber();

boxsize = Dialog.getChoice();

// set box color for different puncta

tmp=puncta_count - floor(puncta_count/3)*3;

if(tmp==1)  colorflag="red";

if(tmp==2)  colorflag="green";

if(tmp==0)  colorflag="blue";

}

// read XY coordinates from the filename_kymoline.txt file.

function readXYcoordinates()

{

tmpname=substring(name,0, indexOf(name, "_kymo.tif") );

tmpname=tmpname+"_kymoline.txt";

data = File.openAsString(dir+tmpname);

datapoints = split(data, "\n");

for (i = 0; i < kymolength; i++)

{

tmp = split(datapoints[i], "\t");

cor_X[i]=parseFloat(tmp[0]);

cor_Y[i]=parseFloat(tmp[1]);

}

}

// set up analyze,

// duplicate kymo image to "select"

// open raw image for showing the actauly track "show"

function start_up()

{

if( File.exists(dir+name+"_select.tif") == 1 )

open(dir+name+"_select.tif");

else

{

// create "select" image"

selectWindow(name);

run("Duplicate...", "title=1"); run("8-bit");

selectWindow(name);

run("Duplicate...", "title=2"); run("8-bit");

selectWindow(name);

run("Duplicate...", "title=3");  run("8-bit");

run("Merge Channels...", "c1=1 c2=2 c3=3");

selectWindow("RGB");

}

rename("select");

// create "show" image

tmpname=substring(name,0, indexOf(name, "_kymo.tif") );

open(dir+tmpname);

selectWindow(tmpname);

run("Fire");

rename("show");

for(i=0;i<5000;i++)

tmpX[i]=-1;

}

//main analyze kymo

function kymo_cal()

{

quit=0;  t_last=-1;

selectImage("select");

//selectImage(ID_show);

start_flag=0;

if(getVersion>="1.37r")

setOption("DisablePopupMenu", true);

while ( quit==0   )

{

// track cursor position

getCursorLoc(x, y, z, flags);

if (x!=x2 || y!=y2 || z!=z2 || flags2!=flags)

{

// shift+left click: use click the point record the position

if (flags&shift!=0  && flags&leftButton!=0 )

{

if( y <= t_last )

{

//print(" please select lager T");

}

// if it is the first click  (start_flag)

if( start_flag==0 )

{

start=y;

//record start y (mean in t dimention)

T[click_index]=y; KP[click_index]=x; click_index++;

start_flag=1;

// reset start_flag

tmpX[start]=x;

// x position for start point

// set color by colorflag

if( colorflag=="red")  setColor(255,0,0);

if( colorflag=="green") setColor(0,255,0);

if( colorflag=="blue")  setColor(0,0,255);

selectImage("select");

temp=floor(boxsize/2);

drawRect(x-temp,y-temp,boxsize,boxsize);    // draw box on user click point the show image

t_last=y;

// last point of track (t_last) set here.

}

// end if( start_flag==0 )

// if it is not he first click, and y (t dimention) is later than the previous t set.

if( y > t_last  )

{

tmpX[y]=x;

// record y(t) and x postion

T[click_index]=y; KP[click_index]=x; click_index++;

if( colorflag=="red")  setColor(255,0,0);

if( colorflag=="green") setColor(0,255,0);

if( colorflag=="blue")  setColor(0,0,255);

selectImage("select");

temp=floor(boxsize/2);

drawRect(x-temp,y-temp,boxsize,boxsize);  //  draw user clicked box on "show"

// interpolate postion between tow user clicked positions

for(i=t_last+1; i<y; i++)

{

tmpX[i] =round( tmpX[t_last]- (i-t_last)*(tmpX[t_last]-tmpX[y])/(y-t_last) );

}

t_last=y;

end=t_last;

// end is the end of the track

}  // end if( y > t_last  )

}  // end  if (flags&shift!=0  && flags&leftButton!=0 )

// ctr0l+right click stop the selection

if (flags&ctrl!=0 && flags&rightButton!=0   )

quit=1;

} // end  if (x!=x2 || y!=y2 || z!=z2 || flags2!=flags)

x2=x; y2=y; z2=z; flags2=flags;

wait(10);

}  // end while ( quit==0   )

// record XY positions for this puncta track to posX and posY

for(i=start;i<=end;i++)

{

posX[i]=cor_X[tmpX[i]];

posY[i]=cor_Y[tmpX[i]];

//drawRect(tmpX[i],i,1,1);

}

// create visual track file in "show";

show_box();

}

// end function

function show_box()

{

selectImage("show");

roiManager("reset");

for(i=start;i<=end;i++)

{

setSlice(i+1);

x=posX[i]; y=posY[i];

makeRectangle(x-3,y-3,7,7);

roiManager("Add");

}

}

// user decide t osave the puncta track.

// draw dots on the puncta track on "select"

// save roi.zip on the "show"

function savepuncta()

{

// draw track dots on "select"

selectImage("select");

if( colorflag=="red")  setColor(255,0,0);

if( colorflag=="green") setColor(0,255,0);

if( colorflag=="blue")  setColor(0,0,255);

for(i=start;i<=end;i++)

{

temp=floor(boxsize/2);

drawRect(tmpX[i]-temp,i-temp,boxsize,boxsize);

}

// save Puncta roi .zip

roiManager("Save", dir+name+"_PunctaTrack_"+puncta_count+".zip");

roiManager("reset");

selectImage("select");

saveAs(dir+name+"_select.tif");

close();

// raw data format is 4 column as

// timepoint - X position - Y position - x position in Kymograph (use to decide move direction)

print("\\Clear");

for(i=0;i<click_index;i++)

print(T[i]+"	"+KP[i]);

selectWindow("Log");

saveAs("Text", dir+name+"_Puncta_"+puncta_count+"_raw.txt");

selectWindow("Log"); run("Close");

}

“Measure”

// Measure Macro

// for user give folder, calcualte on all .txt file.

// txt file should have 2 column as below seperate by tab (generated by Track Sunny macro

// timepoint - Kymp Point

// .txt file are all pixel unit, user input xyscale and timescale

// resultfile save in teh same folder

// create track_roi to show all calucalted tracks

// initialize ImageJ

run("Colors...", "foreground=white background=black selection=yellow");

run("Options...", "iterations=1 count=1 black edm=Overwrite");

// parameters

// pixelscale : ?? um/pixel   timegap : time between each frame   speed_limitL : lowest speed limit, if speed lower than this, consider puncta still

var dir, name, pixelscale, timegap, speed_limitL, kymoX, kymoY, track_index, track_name;

// T KP position of each user clcik track point

var T=newArray(500), KP=newArray(500);

var count;

//move direction in Kymo graph

var dirX=newArray(500);

//------------------------------- start main function -------------------------------------

dir=getDirectory("Choose Data Folder");

list=getFileList(dir);

// input analyze parameters

parameterinput();

//print header for analysis result file for this puncta

printheader();

// create image to store all kymo track

newImage("track", "8-bit black", kymoX, kymoY, 1);

track_index=0;

for(f=0; f<list.length;f++)

{

name=list[f];

if( indexOf(name, ".txt") >0 )

{

track_name = substring(name, indexOf(name, "_Puncta_")+8, indexOf(name, "_raw.txt"));

// read into each timepoints with X Y position and Kymo x position

// and create kymo track roi

readParameters();

// parameter calculated. _t means total time point in this category, _seg means total segment # in this category

move_t=0; move_seg=0; still_t=0; still_seg=0; move_length=0;

ant_t=0; ant_seg=0; ant_length=0; ant_per=0;

retro_t=0; retro_seg=0; retro_length=0; retro_per=0;

reverse=0; stop=0;

// t0, kp0 are the previous position    dir0 is last direction    0: still  1:ant   -1:retro

// in the loop x1, y1 is the current position  pre_dir is the previous moving direction;

t0=T[0]; kp0=KP[0]; dir0=0;  pre_dir=0;

// for every cal_gap points,  reall index in raw data is i*cal_gap

for(i=1; i< count; i++)

{

// t1 kp1 is the current position

t1=T[i]; kp1=KP[i];

timecount=t1-t0;  // timecount is how many times points in this segment

length=abs(kp1-kp0) * pixelscale;   // length is total length (um) in this segment

if( kp1 > kp0 ) dir1=1;   // get direction information of this segment

if( kp1 < kp0 ) dir1=-1;

// calculate speed in this segment in um/s unit

speed=( ( abs(kp1-kp0)*pixelscale ) / ( (t1-t0) * timegap) );

//compare to speedlimit

// moving segment

if( speed > speed_limitL )

{

move_seg++;  move_t=move_t+timecount; move_length=move_length+length;    // move# cal

if( dir1 == 1 )  { ant_seg++; ant_t=ant_t+timecount; ant_length=ant_length+length; }  // ant# cal

if( dir1 == -1 )  { retro_seg++; retro_t=retro_t+timecount; retro_length=retro_length+length;  } // ant# cal

if( abs( dir1 - pre_dir) > 1.5 )  reverse++;   // reverse cal

pre_dir=dir1;

}

// still segment

if( speed <= speed_limitL )

{

dir1=0;

still_seg++;  still_t=still_t+timecount;   // still# cal

if( dir0 != 0 ) stop++;

}

t0=t1; kp0=kp1; dir0=dir1;

}  // end for(i=1; i< count; i++)

// percentage

totaltime=T[count-1]-T[0];

moveF=move_t/totaltime; stillF=still_t/totaltime; antF=ant_t/totaltime;  retroF=retro_t/totaltime;

moveS = 0;

if( move_t > 0 )  moveS=move_length/ ( move_t * timegap);

antS=0;

if( ant_t > 0 )  antS=ant_length/ ( ant_t * timegap);

retroS=0;

if(retro_t > 0 ) retroS=retro_length/ ( retro_t * timegap);

aveSegT=0;

if( move_seg > 0 )  aveSegT=move_length/ move_seg;

aveantSegT=0;

if( ant_seg > 0 )  aveantSegT=ant_length/ ant_seg;

averetroSegT=0;

if( retro_seg > 0 )  averetroSegT=retro_length/ retro_seg;

//print("#	StartTimePoint	EndTimePoint	TotalTime	overall	ant	retro	still	overall	ant	retro	overall	ant	retro	overall	ant	retro	overall	ant	retro	still	reverse#	stop#");

print(list[f]+"	"+T[0]+"	"+T[count-1]+"	"+move_seg+still_seg+"	"+moveF+"	"+antF+"	"+retroF+"	"+stillF+"	"+moveS+"	"+antS+"	"+retroS+"	"+move_length+"	"+ant_length+"	"+retro_length+"	"+aveSegT+"	"+aveantSegT+"	"+averetroSegT+"	"+move_seg+still_seg+"	"+ant_seg+"	"+retro_seg+"	"+still_seg+"	"+reverse+"	"+stop);

//for(i=0; i< count-1; i++)

//makeLine(KP[i],T[i], KP[i+1],T[i+1]);

//roiManager("Add");

} // end if( indexOf(name, ".txt") >0 )

} // end for(f=0; f<list.length;f++)

selectWindow("Log");

saveAs("Text", dir+"Summary.xls");

run("Close");

roiManager("Save", dir+"Track_ROI.zip");

roiManager("reset");

selectImage("track");

close();

selectWindow("ROI Manager");

run("Close");

//---------------------------------------  function line ------------------------------------------------------

// input parameters for calculation

function parameterinput()

{

Dialog.create("Parameter");

Dialog.addNumber("PixelScale (um/pixel):", 0.1953);

Dialog.addNumber("TimeScale (s):", 1);

Dialog.addNumber("Speed Limit Low(um/s):", 0.05);

Dialog.addNumber("Kymograph X:", 100);

Dialog.addNumber("Kymograph Y:", 100);

Dialog.show();

pixelscale = Dialog.getNumber();

timegap = Dialog.getNumber();

speed_limitL = Dialog.getNumber();

kymoX = Dialog.getNumber();

kymoY = Dialog.getNumber();

}

// read XY coordinates and x pisition on kymo fro raw txt file

function readParameters()

{

data = File.openAsString(dir+name);

datapoints = split(data, "\n");

count=datapoints.length;

for (i = 0; i < datapoints.length; i++)

{

tmp = split(datapoints[i], "\t");

T[i]=parseFloat(tmp[0]);

KP[i]=parseFloat(tmp[1]);

if( i>0 )

drawLine( KP[i-1], T[i-1], KP[i], T[i]);

}

selectImage("track");

run("Analyze Particles...", "size=0-infinity add");

roiManager("Select", track_index);

roiManager("Rename", track_name);

roiManager("Select", track_index);

roiManager("Deselect");

run("Select None");

run("Multiply...", "value=0");

track_index++;

}

// print Log window header for each pucnta track

function printheader()

{

print("\\Clear");  // clear Log window

print("TrackRawFileFolder:	"+dir);

print("XY Unit(um):	"+pixelscale+"	T Unit(s):	"+timegap+"	Speed_Limit_Low(um/s):	"+speed_limitL);

print("\n");

print("Puncta#	TIME	 	 	% Time Traveld(%)	 	 	 	Speed(um/s)	 	 	Total Distance Traveld (um)	 	 	Ave Segment Traveld (um)	 	 	# of Segments	 	 	 	Reversed	Stop");

print("#	StartTimePoint	EndTimePoint	TotalSegment	overall	ant	retro	still	overall	ant	retro	overall	ant	retro	overall	ant	retro	overall	ant	retro	still	#	#");

}