Supplemental Information – Gas Manifold

- Make the gas manifolds from metal with airtight seals that can withstand ~100 psi as a working pressure to make anaerobic media.

- Use Swagelok gas tight fittings as these have proven to be the most reliable and economical.

- Make sure to have the proper regulator on the nitrogen gas tank (if using a different gas type, make sure the appropriate regulator is affixed to it before using).

- Use pipe fitting Teflon tape to improve airtight seals when connecting using “pipe fitter standard” tapering threads.

- Use pressure gauges to allow trouble shooting and verification of gas pressure within the closed system.