Commented Arduino sketch for water speed measurements.

Comments, for each line of code, are included after the two forward slashes (//).

#include <LiquidCrystal.h>

LiquidCrystal lcd(7,8,9,10,11,12); // initialize the liquid crystal display with the indicated pins (rs, en, d4, d5, d6, d7), R/W pin to ground, VSS pin to ground, VCC pin to 5V, 10kohm potentiometer ends connected to 5V and ground, wiper to LCD V0, A pin to 5V, K pin to GND.

volatile int Count; //measuring the rising edges of the signal

int Calc; //calculation of flow rate in L/h passing through the sensor

float vel; //speed of water in m/s

float Time; //time since turning on the Arduino

int hallsensor = 2;    //The pin location (Digital2) of the sensor. Sensor wire is connected to a 10kohm pullup resistor connected to 5V and then connected to Digital2.

void rpm ()     //Function called by the interrupt

{

Count++;  //Add 1 to the count every time the signal of the hall sensor is rising (e.g., every time the flapper completes one turn)

}

void setup ()

{

lcd.begin(16, 2);

pinMode(hallsensor, INPUT);

attachInterrupt(0, rpm, RISING); //interrupt 0 corresponds to digital pin 2 on the Arduino Nano. Call the interrupt function (rpm) when hall sensor signal from the flapper rises.

}

void loop() {

Count = 0;      //Set Count to 0 to begin calculations

sei();            //Enable interrupts

delay (1000);      //Wait 1 second while interrupt is triggered by RISING signal, resulting in the addition of 1 to the count every time the flopper completes 1 turn, obtaining the number of turns per second.

cli();            //Disable interrupts

Calc = (Count * 60 / 6.6); //(Pulse frequency x 60) / 6.6Q, = flow rate in L/h, Q denotes how many L per minute corrensponds to 1 turn of the flapper (Q corresponds to 6.6 for the YF-B5 sensor, HaiHuiLai, China, 3/4'' brass water flow sensor Hall effect NPN pulse)

vel = Calc/1246.86; //speed of water in m/s, pipe section 2.1 cm diameter (cross-sectional area 0.00034635 m2, obtained after multiplying the square of the radius by pi).

//Liters per hour were transformed to cubic meters per second and divided by the section area to obtain water speed in m/s (vel=Q/cross-sectional Area).

Time = millis()/60000.0; //calculate Time in minutes

lcd.clear();

lcd.setCursor (0,0);

lcd.print ("L/h=");

lcd.setCursor (4,0);

lcd.print (Calc, DEC); //print  flow rate in L/h

lcd.setCursor (9,0);

lcd.print ("min=");

lcd.print (Time, DEC); //print  time since turning on the Arduino in minutes

lcd.setCursor (0,1);

lcd.print ("m/s=");

lcd.setCursor (4,1);

lcd.print (vel, DEC); //print  the speed of water in m/s

}