Supplementary File 1

Design and lead weight testing for remote ischemia-reperfusion injury

Lead weights have been selected to keep traction on the sutures during inflation and deflation. The advantage of this open chain system is that too extensive pressure cannot be exerted on the LAD as this depends on the weight of the lead. Therefore, ex-vivo testing using a spring balance dynamometer has been performed. The dynamometer was connected to the remote IRI setup and tested with various weights. The same dynamometer was then tested with the original pulley system1. The original pulley system used a weight of 4 grams on both sides to induce ischemia. This correlated with a weight of 2.1 g of lead in the proposed remote IRI tool (Supplementary Figure 1).

A 500 mm x 300 mm polymethyl methacrylate (PMMA) plate with a thickness of 2 mm is used as the base material for the construction of the tool. The base plate and side vertical plate were cut with a laser cutter based on the measurements provided. The presented design allows easy adjustments to the tool based on the PMMA thickness (measurements X and Y) and the length of the balloon catheter (number, size, and spacing of the balloon anchor points) (Supplementary Figure 2).

In vivo MRI setup

Anesthesia was induced with 3% isoflurane in 100% O2 in an induction chamber, followed by maintenance at 1.5%–2% isoflurane at 1.5 L/min O2 via nose cone. Mice were positioned supine in a purpose-built cradle and secured with surgical tape. Body temperature, respiration, and cardiac rate were monitored and maintained at 36–37 °C, 70–100 min-1, and 450–600 min-1, respectively. The core temperature was maintained with warm water in a closed circuit within the cradle. ECG signal was derived from needles inserted subcutaneously in the front paws. Respiratory monitoring was performed with a balloon pressure transducer positioned on the animal's abdomen, and temperature monitoring was performed with a rectal temperature probe. Cardio-respiratory monitoring was performed using an MR-compatible system.

LGE CMR Image Acquisition

MRI experiments were performed on a 7T small animal MRI system equipped with actively shielded gradients (200 mT/m). An 86 mm quadrature transmit coil was used for radiofrequency transmission, and a surface receive-only coil was placed over the chest of the animal. The contrast was delivered via an intraperitoneal injection of 80 μL total volume with 0.5 mmol/ kg Gd-DOTA diluted in warm saline to the right abdominal quadrant. The animal cradle was then positioned in the center of the MRI. Cardiac planning was performed using three-plane cardiac scout images acquired with a non-gated fast gradient echo imaging sequence. Semi-2 chamber (2CH), semi-4 chamber (4CH), and short-axis (SAX) images were then acquired. The 2/4CH images were used to plan the subsequent stack of cine SAX images, which was acquired ~20 min post-injection. A self-gated gradient echo sequence in axial orientation was acquired with the following parameters: TE, 2.3 ms, TR, 11.2 ms, flip angle, 14°, matrix, 150 x 150, FOV, 30 mm x 30 mm, 7–8 contiguous slices of 1 mm thickness, oversampling: 200. The total image acquisition time was approximately 45 min. Images were reconstructed using the ParaVision software to 20 cardiac frames excluding respiration motion.

Image analysis

MRI data were analyzed by a single investigator in a random order. Bias field correction was performed using a custom MATLAB2 script based on code available at Chunming et al.3. Next, endocardial and epicardial borders of the left ventricle in end-diastole were delineated using Segment v4.04. Area at risk (AAR) region of interest (ROI) was quantified in 3 apical slices (excluding the apex itself) using a threshold-based approach of raw myocardial signal intensity (SI). Manual ROIs were drawn in the septal area to represent the viable myocardium. The software then delineated the AAR from the remaining part of the myocardium where signal intensities exceeded the threshold of 2 standard deviations (SD) from viable ROI.

Area At Risk=slicesN pixel (SI>2SD)N pixel myocardium*100%

References

1. Oosterlinck, W. et al. Diabetes mellitus and the metabolic syndrome do not abolish, but might reduce, the cardioprotective effect of ischemic postconditioning. J Thorac Cardiovasc Surg. 145 (6), 1595–1602 (2013).

2. The MathWorks Inc. MATLAB version: 9.10.0.1739362 (R2021a) Update 5, Natick, Massachusetts: The MathWorks Inc. https://www.mathworks.com.

3. Chunming, Li. MRI segmentation and bias field correction (https://www.mathworks.com/matlabcentral/fileexchange/59752-mri-segmentation-and-bias-field-correction), MATLAB Central File Exchange.

4. Heiberg, E. et al. Design and validation of Segment - freely available software for cardiovascular image analysis. BMC Med. Imaging. 10 (1), 1 (2010).