SUPPLEMENTARY PROTOCOLS

- Spheroid formation methods

  - Micro-patterned agarose-coated tissue culture plates (= lab-made microwells)

NOTE: This high throughput method has already been published1 and is used to simultaneously generate a high number of spheroids (1589 spheroids) in microwells of 400 µm diameter. The following protocol has been modified to avoid damaging the agarose, introducing air bubbles, and increasing reproducibility.

    - Prepare all sterile materials and instruments (spatulas and tweezers) by autoclaving where possible or by filter sterilization. Clean the recyclable PDMS stamps from agarose and store them aseptically in 70% ethanol.

NOTE: It is suggested that this be done at least 1 day before spheroid generation.

    - Transfer micro-patterned PDMS stamps from a storage vial to a sterile Petri dish and air-dry with the smooth surface up under the sterile laminar airflow conditions for 10 min.

    - Put each stamp with a micro-patterned surface up (and smooth surface down) in the middle of the well of a 6-well sterile tissue culture plate. Air dry for 1-2 min with an open lid.

NOTE: It is highly important to evaporate the excess liquid as only dry stamps perfectly stick to the plastic surface and remain attached during the procedure.

    - Weigh 1.5 g of agarose into a clean 200 mL glass bottle, add 50 mL of sterile PBS, cover with a lid, and melt in a microwave to make a 3% homogeneous agarose solution.

CAUTION: The agarose solution is extremely hot and has to be handled with caution. If shaken immediately after the melting procedure, the hot agarose solution can start bubbling and burst out of the vessel. To avoid occasional traumas, use sufficiently large vessels filled with a maximum of 50% of the volume with the agarose solution and use appropriate personal protection (gloves, oven mitt, eye protection, and lab coat).

    - Using a sterile serological pipette, immediately add ~7 mL of hot agarose solution to each well of the 6-well cell culture plates to completely cover the inserted PDMS stamp. Leave the agarose to solidify for 20 min by incubating under the sterile airflow with the plate lid opened.

NOTE: The agarose layer should be at least two times thicker than the PDMS stamp to ensure the integrity of the agarose microwells after the PDMS stamp removal.

    - Add 1 mL of sterile PBS + 2x Pen/Strep on the agarose before using a sterile small spatula to accurately turn the agarose with the embedded PDMS stamp upside down in each well to have the smooth stamp surface up.

NOTE: The addition of PBS will make the agarose easier to handle, hereby avoiding damaging.

    - Add 500 µL of PBS + 2x Pen/Strep on the smooth topside of the stamp if needed, gently detach the stamp from the agarose with a spatula, and remove it from the well. Avoid damaging the microwells.

    - Remove as much PBS as possible from the molds and add 3–4 mL of corresponding sterile cell culture media to the agarose stamps for direct use. Cover the plate with the lid and incubate for at least 3 days in the incubator with daily refreshments. Do not let the agarose dry.

NOTE: Use PBS solution (instead of medium) for long-term storage (up to 2 weeks at 4 °C). Incubation is needed to equilibrate and remove air bubbles from agarose microwells. Due to the thickness of the agarose, equilibration in the culture media can take up to 3 days with daily refreshments.

    - Before use, remove 1 mL of old media (or PBS) and add 1 mL of cell suspension containing 7,94,500 cells premixed with the O2 probe (step 1.1 of the main protocol).

NOTE: The number of micro-wells in agarose stamps determines the number of spheroids produced per stamp. In this case, this agarose mold contains 1589 microwells (1589 microwells x 500 cells/spheroid). Variation of cell concentration in suspension added to a macrowell allows changing the cell number per spheroid and controlling their size.

    - Culture for 5 days to ensure spheroid formation and compaction.

NOTE: Avoid over-drying of spheroid culture. If necessary, provide an additional amount of culture media containing a diluted O2 probe and avoid disturbing the spheroids in micro-wells.

  - SphericalPlate 5D

NOTE: This high throughput method is used to simultaneously generate a high number of spheroids (750 spheroids per well) in an integrated grid forming micromolds with 90 µm diameter.

    - Add 1 mL of growth media to each well.

    - Centrifugate at 1,000 x g for 2 min to remove trapped air bubbles.

NOTE: Confirm this visually under a microscope; otherwise, repeat this step.

    - Put the plate in the incubator at 37 °C while cell suspension is prepared (see step 1.1 of the main protocol).

    - Add 1 mL of cell suspension containing 3,75,000 cells dropwise.

NOTE: The number of micro-wells integrated into the well determines the number of spheroids produced per stamp. In this case, one well contains 750 microwells (750 x 500 cells/spheroid). Variation of cell concentration in suspension added to a macrowell allows changing the cell number per spheroid, thus controlling the spheroid's size.

    - Allow ~10 min for cells to settle into the microwell grids.

    - Place the tissue culture plate in the cell culture incubator and exchange media as needed.

REFERENCES:

- Okkelman, I. A., Vercruysse, C., Kondrashina, A. V., Borisov, S. M., Dmitriev, R. I. Affordable oxygen microscopy-assisted biofabrication of multicellular spheroids. J Vis Exp. 182, e63403 (2022).