Supplementary File 2: Microscope illumination settings

| Type | Illumination cycle | Illumination pattern | Script |
| --- | --- | --- | --- |
| Pulsed Global Imaging for FastLightR-Src and FastLightR-bRaf with Ring Light | 10 min basal dark>30 min Light>20min Dark>30 min Light>20 min Dark | 12s Continuous with Time course of 4 cells/ minute | Main Script: IF (MDA Status.TimePointNum >= 11) and (MDA Status.TimePointNum <= 41) or (MDA Status.TimePointNum >= 61) and (MDA Status.TimePointNum <= 91) THEN - Loop a Journal(1, NOPROMPT, “”, “Sh2 open-close pulse”) ELSE - Delay(12, SECONDS) END IF ***End of Journal*** Secondary Script (Sh2 open-close pulse): Component Control(Component Name) = NI-DAQmx Shutter 2, Position = -1) Delay(12, SECONDS) Component Control(Component Name) = NI-DAQmx Shutter 2, Position = 0) |
| Local Imaging for Fast LightR-Src with Micro Mirror System | 20 min basal dark>50 min Light>20 min Dark | 50 s Continuous with Time course of 1 cell/minute | Main Script: IF (MDA Status.TimePointNum >= 21) and (MDA Status.TimePointNum <= 71) THEN - Select Illumination(“Polygon”) - Delay(50, SECONDS) ELSE - Delay(50, SECONDS) END IF ***End of Journal*** |
| Global Imaging for Fast LightR-bRaF and Fast LightR-Src Using 445nm Epifluorescence | 10 min basal dark>30 min Light>20min Dark>30 min Light>20 min Dark | 50 sec/ minute illumination; 50 pulses/Sec with Time course of 1 cell/ minute | Main Script: IF (MDA Status.TimePointNum >= 11) and (MDA Status.TimePointNum <= 41) or (MDA Status.TimePointNum >= 61) and (MDA Status.TimePointNum <= 91) THEN - Loop a Journal(50, NOPROMPT, “”, “single illumination_445laser”) ELSE - Delay(50, SECONDS) END IF ***End of Journal*** Secondary Script (single illumination_445laser): - Select Illumination(“GFP CAM”) - Open Shutter - Delay(50, MILLISEC) - Close Shutter ***End of Journal*** |