## Pseudocode of the script used for running the tomography measurements with the sample changer robot

TOMOMATIC BATCH SCAN

- Program to run a series of tomography scans using a robotic sample changer.

- INPUT: CSV file containing details of individual samples and their corresponding scan parameters.

- Parse the input CSV file to generate an initial list of sample and scan parameter pairs (i.e., sample, scan parameters).

- Sort the initial list of sample and scan parameter pairs by input priority to generate the final ordered list.

- Check the robotic arm is safe to operate.

- FOR each (sample, scan parameters) in the ordered list

- Instruct the robot to remove the current sample from the scanning position on the rotation stage, take the next sample to the Data matrix code reader.

- Instruct the robot to move the next sample into the scanning position.

- FOR i < max number of attempts at sample centring

- Run sample centring for the current sample.

IF sample centring = SUCCESS

Run the tomography scan using the scan parameters associated with the current sample.

BREAK

END FOR

END FOR