ANNEX

Exemplary user-defined functions (UDF):

UDF defining laserspot and power intake:

# include ”udf . h”

DEFINE_SOURCE(mw40mirkom300 , c , t , ds , eqn )

{

real radius ;

real power ;

//power of the laser (in Watts)

real transmission ;

real absorbRPE ;

real a ;

real powerLocal ;

real distance ;

//distance between calculated position and center of a cell

real x [ND_ND] ;

// this will hold the position vector

real source ;

//currentTimeStep = N_TIME;

//N_TIME i s a Macro , returning the current timestep

power = 0.2 ;

radius = 0.00015 ;

transmission = 0.2 ;

absorbRPE = 0.51 ;

a = 27000;

C_CENTROID(x ,c,t) ;

//sets x−vector (3D) to the position of the current cell

distance = x[0] * x[0] + x[2] * x[2] ;

powerLocal = power* transmission/radius/radius /3.14159* exp(− distance/radius/radius ) ;

if (x[1] > −0.00001 && x[1] < −0.000005) {

source = powerLocal* absorbRPE /0.000005

}

else {

if ( x[1] > 0) {

source = a * powerLocal*(1−absorbRPE ) * exp(−a * x[1] ) ;

}

else {

source = 0 ;

}

}

return source ;

}

UDF scaling the blood flow vector field:

# include ”udf . h”

//# include <math . h>

DEFINE_ON_DEMAND( VelocityScale )

{

real betragIst ;

real betragSoll ;

real skalierung ;

real u ;

real v ;

real w;

Domain *d ;

Thread * t ;

cell _t c ;

d = Get_Domain ( 1 ) ;

t = Lookup_Thread (d , 2 7 ) ;

begin_c_loop ( c , t )

{

u = C_U( c , t ) ;

v = C_V( c , t ) ;

w = C_W( c , t ) ;

b e t r a g I s t = sqrt (u*u +v*v + w*w) ;

betragSoll = 12345;

skalierung = betragSoll/betragIst ;

C_U( c , t ) = u* skalierung ;

C_V( c , t ) = v* skalierung ;

C_W( c , t ) = w* skalierung ;

}

end_c_loop ( c , t )