Code of fan in LabVIEW Formula Node:

SteadyRPM = KMaxRPM * u * (1 - 0.6 * Load);

SteadyRPM = SteadyRPM - KMaxRPM * Disturbance * (0.15 + 0.10 * sin(0.8 * time_prev));

if (SteadyRPM < 0)

SteadyRPM = 0;

speed_next = speed_prev + (0.05 / Tau) * (SteadyRPM - speed_prev);

if (speed_next < 0)

speed_next = 0;

if (speed_next > KMaxRPM)

speed_next = KMaxRPM;

SpeedNorm = speed_next / KMaxRPM;

CurrentA = 0.08 + 0.35 * u + 0.08 * Disturbance;

PowerW = 12 * CurrentA;

time_next = time_prev + 0.05;

Code of Motor in LabVIEW Formula Node:

float64 R;

float64 L;

float64 J;

float64 b;

float64 Kt;

float64 Ke;

float64 Vmax;

float64 e;

float64 d_e;

float64 i_term;

float64 d_term;

float64 u;

float64 u_total;

float64 di;

float64 domega;

R = 1.0;

L = 0.5;

J = 0.01;

b = 0.1;

Kt = 0.01;

Ke = 0.01;

Vmax = 24.0;

if (reset != 0) {

theta_next = 0.0;

omega_next = 0.0;

im_next = 0.0;

e_next = 0.0;

integ_next = 0.0;

voltage = 0.0;

}

else {

e = sp - theta;

integ_next = integ + e * dt;

d_e = (e - e_prev) / dt;

if (Ti <= 0.0) {

i_term = 0.0;

}

else {

i_term = integ_next / Ti;

}

if (Td <= 0.0) {

d_term = 0.0;

}

else {

d_term = Td * d_e;

}

u = Kc * (e + i_term + d_term);

if (u > Vmax) {

u = Vmax;

}

if (u < -Vmax) {

u = -Vmax;

}

voltage = u;

u_total = voltage + disturbance;

di = (u_total - R * im - Ke * omega) / L;

domega = (Kt * im - b * omega) / J;

im_next = im + di * dt;

omega_next = omega + domega * dt;

theta_next = theta + omega * dt;

e_next = e;

}